/ u0 -- unix

cold = 0

orig = 0 . / orig = 0. relocatable

rkda = 177412 / disk address reg            rk03/rk11

rkds = 177400 / driv status reg             rk03/rk11

rkcs = 177404 / control status reg          rk03/rk11

rcsr = 174000 / receiver status reg         dc-11

rcbr = 174002 / receiver buffer reg         dc-11

tcsr = 174004 / xmtr status reg             dc-11

tcbr = 174006 / xmtr buffer reg             dc-11

tcst = 177340 / dec tape control status     tc11/tu56

tccm = 177342 / dec tape command reg        tc11/tu56

tcwc = 177344 /          word count         tc11/tu56

tcba = 177346 /          bus addr           tc11/tu56

tcdt = 177350 /          data reg           tc11/tu56

dcs  = 177460 / drum control status         rf11/rs11

dae  = 177470 / drum address extension      rf11/rs11

lks  = 177546 / clock status reg            kw11-l

prs  = 177550 / papertape reader status     pc11

prb  = 177552 /                  buffer     pc11

pps  = 177554 /            punch status     pc11

ppb  = 177556 /            punch buffer     pc11

/lps = 177514   line printer status         (future)

/lpb = 177516   line printer buffer         (future)

tks  = 177560 / console read status         asr-33

tkb  = 177562 /         read buffer         asr-33

tps  = 177564 /         punch status        asr-33

tpb  = 177566 /         punch buffer        asr-33

ps   = 177776 / processor status

halt = 0

wait = 1

rti  = 2

nproc = 16. / number of processes

nfiles = 50.

ntty = 8+1

nbuf = 6

.if cold / ignored if cold = 0

nbuf = 2

.endif

core = orig+40000  / specifies beginning of user's core

ecore = core+20000 / specifies end of user's core (4096 words)

/      4;4     init by copy

/      unkni;0 " error

/      fpsym;0 " illg in tr

unkni;0 / trace and trap (see Sec. B.1 page  )

unkni;0 / trap

panic;0 / pwr

rtssym;0 / emt

sysent;0 / sys

. = orig+60

ttyi;240 / interrupt vector tty in       ; processor level 5

ttyo;240 / interrupt vector tty out

ppti;240 /                  punch papertape in

ppto;240 /                  punch papertape out

clock;340 / clock interrupt vector       ; processor level 7

. = orig+200

/     lpto; 240  line printer interrupt     ; processor level 5 (future)

. = orig+204

drum;300 / drum interrupt         ; processor level 6

. = orig+214

tape;300 / dec tape interrupt

disk;300 / rk03 interrupt

. = orig+300

0*4+trcv; 240; 0*4+txmt; 240 / dc11 input,output interrupt vectors

1*4+trcv; 240; 1*4+txmt; 240

2*4+trcv; 240; 2*4+txmt; 240

3*4+trcv; 240; 3*4+txmt; 240

4*4+trcv; 240; 4*4+txmt; 240

5*4+trcv; 240; 5*4+txmt; 240

6*4+trcv; 240; 6*4+txmt; 240

7*4+trcv; 240; 7*4+txmt; 240

. = orig+400

/ copy in transfer vectors

mov    $ecore,sp / put pointer to ecore in the stack pointer

jsr    r0,copyz; 0; 14 / clear locations 0 to 14 in core

mov    $4,r0

clr    r1

mov    r0,(r1)+ / put value of 4 into location 0

mov    r0,(r1)+ / put value of 4 into location 2

mov    $unkni,(r1)+ / put value of unkni into location 4;

/ time out, bus error

clr    (r1)+ / put value of 0 into location 6

mov    $fpsym,(r1)+ / put value of fpsym into location 10

clr    (r1)+ / put value of 0 into location 12

/ clear core

.if cold / ignored if cold = 0

halt / halt before initializing rf file system; user has

/ last chance to reconsider

.endif

jsr    r0,copyz; systm; ecore / clear locations systm to ecore

mov    $s.chrgt+2,clockp / intialize clockp

/ allocate tty buffers; see H.0 for description

mov    $buffer,r0

mov    $tty+6,r1

1:

mov    r0,(r1)

add    $140.,r0 / tty buffers are 140. bytes long

add    $8,r1

cmp    r1,$tty+[ntty*8] / has a buffer been assigned for each tty

blo    1b

/ allocate disk buffers; see H.0 for description

mov    $bufp,r1

1:

mov    r0,(r1)+

add    $8,r0

mov    r0,-2(r0)             / bus address

mov    $-256.,-4(r0)         / word count

add    $512.,r0              / buffer space

cmp    r1,$bufp+nbuf+nbuf

blo    1b

mov    $sb0,(r1)+            / I/O queue entry drum

mov    $sb1,(r1)+ / I/O queue entry disk (mounted device)

mov    $swp,(r1)+ / I/O queue entry core image being swapped

mov    $[systm-inode]\/2,sb0+4 / sets up initial buffers per

/ format given in

mov    $systm,sb0+6 / memory map

mov    $-512.,sb1+4

mov    $mount,sb1+6

mov    $user,swp+6

/ set devices to interrupt

mov    $100,*$lks / put 100 into clock status register;

/ enables clock interrupt

/ set up time out subroutines

mov    $touts,r0

mov    $startty,(r0)+ / if toutt = 0 call startty

mov    $pptito,(r0)+ / if toutt+1 = 0 call pptito

tst    (r0)+ / add 2 to r0

mov    $ntty-1,r1

1 :

mov    $xmtto,(r0)+ / if toutt+2 thru toutt+2+ntty=0 call xmtto

dec    r1

bne    1b

/ free all character blocks; see H.0 for description

mov    $510.,r2

mov    $-1,r1

1:

jsr    r0,put

sub    $2,r2

bgt    1b

/ set up drum swap addresses; see H.0 for description

mov    $1024.-64.,r1 / highest drum address; high 64 blks allocated

/ to UNIX

mov    $p.dska,r2 / p.dska contains dis addresses for processes

1 :

sub    $17.,r1 / 17 blocks per process

mov    r1,(r2)+

cmp    r2,$p.dska+nproc+nproc

bne    1b

/ free rest of drum

.if cold

mov    $128.,systm / initialize word 1 of drum superblock image;

/ number of bytes in free storage map=128.

mov    $64.,systm+2+128. / init. wd 66. of superblock image; # of

/ bytes in i-node map=64.

1:

dec    r1 / r1=687.,...,34.

jsr    r0,free / free block 'r1', i.e., set bit 'r1' in free

/ storage map in core

cmp    r1,$34. / first drum address not in i list

bgt    1b / if block 34 has been freed, zero i list

/ zero i list

1:

dec    r1 / r1=33.,...,1

jsr    r0,clear / zero block 'r1' on fixed head disk

tst    r1

bgt    1b / if blocks 33,...,1 have all been zeroed, done.

.endif

/ make current program a user

mov    $41.,r0 / rootdir set to 41 and never changed

mov    r0,rootdir / rootdir is i-number of root directory

mov    r0,u.cdir / u.cdir is i-number of process current directory

mov    $1,r0

movb   r0,u.uno / set process table index for this process to 1

mov    r0,mpid / initialize mpid to 1

mov    r0,p.pid / p.pid identifies process

movb   r0,p.stat / process status = 1 i.e., active

/                = 0 free

.if cold         /                = 2 waiting for a child to die

/                = 3 terminated but not yet waited

/                  for

/ initialize inodes for special files (inodes 1 to 40.)

mov    $40.,r1 / set r1=i-node-number 40.

1:

jsr    r0,iget / read i-node 'r1' from disk into inode area of

/ core and write modified inode out (if any)

mov    $100017,i.flgs / set flags in core image of inode to indi-

/ cate allocated, read (owner, non-owner),

/ write (owner, non-owner)

movb   $1,i.nlks / set no. of links = 1

movb   $1,i.uid / set user id of owner = 1

jsr    r0,setimod / set imod=1 to indicate i-node modified, also

/ stuff time of modification into i-node

dec    r1 / next i-node no. = present i-node no.-1

bgt    1b / has i-node 1 been initialized; no, branch

/ initialize i-nodes r1.,...,47. and write the root device, binary, etc.,

/ directories onto fixed head disk. user temporary, initialization prog.

mov    $idata,r0 / r0=base addr. of assembled directories.

mov    $u.off,u.fofp / pointer to u.off in u.fofp (holds file

/ offset)

1:

mov    (r0)+,r1/r1=41.,...,47; "0" in the assembled directory

/ header signals last

beq    1f      / assembled directory has been written onto drum

jsr    r0,imap / locate the inode map bit for i-node 'r1'

bisb   mq,(r2) / set the bit to indicate the i-node is not

/ available

jsr    r0,iget / read inode 'r1' from disk into inode area of

/ core and write modified i-node on drum (if any)

mov    (r0)+,i.flgs / set flags in core image of inode from

/ assembled directories header

movb   (r0)+,i.nlks / set no. of links from header

movb   (r0)+,i.uid / set user id of owner from header

jsr    r0,setimod / set imod=1 to indicate inode modified; also,

/ stuff time of modification into i-node

mov    (r0)+,u.count / set byte count for write call equal to

/ size of directory

mov    r0,u.base / set buffer address for write to top of directory

clr    u.off / clear file offset used in 'seek' and 'tell'

add    u.count,r0 / r0 points to the header of the next directory

jsr    r0,writei / write the directory and i-node onto drum

br     1b / do next directory

.endif

/ next 2 instructions not executed during cold boot.

bis    $2000,sb0 / sb0 I/O queue entry for superblock on drum;

/ set bit 10 to 1

jsr    r0,ppoke / read drum superblock

1:

tstb   sb0+1 / has I/O request been honored (for drum)?

bne    1b / no, continue to idle.

1:

decb   sysflg / mormally sysflag=0, indicates executing in system

sys    exec; 2f; 1f / generates trap interrupt; trap vector =

/ sysent; 0

br     panic / execute file/etc/init

1:

2f;0

2:

</etc/init\0> / UNIX looks for strings term, noted by nul\0

panic:

clr    ps

1:

dec    $0

bne    1b

dec    $5

bne    1b

jmp    *$173700 / rom loader address

rtssym:

mov    r0,-(sp)

mov    r1,-(sp)

mov    4(sp),r0

mov    -(r0),r0

bic    $!7,r0

asl    r0

jmp    *1f(r0)

1:

0f;1f;2f;3f;4f;5f;badrts;7f

0:

mov    2(sp),r0

br     1f

2:

mov    r2,r1

br     1f

3:

mov    r3,r1

br     1f

4:

mov    r4,r1

br     1f

5:

mov     r5,r1

br     1f

7:

mov    8.(sp),r1

1:

cmp    r1,$core

blo    badrts

cmp    r1,$ecore

bhis   badrts

bit    $1,r1

bne    badrts

tst    (r1)

beq    badrts

add    $1f,r0

mov    r0,4(sp)

mov    (sp)+,r1

mov    (sp)+,r0

rti

1:

rts    r0

rts    r1

rts    r2

rts    r3

rts    r4

rts    r5

rts    sp

rts    pc

badrts:

mov    (sp)+,r1

mov    (sp)+,r0

rpsym:

jmp    unkni

.if cold

idata:

/ root

41.

140016

.byte 7,1

9f-.-2

41.

<..\0\0\0\0\0\0>

41.

<.\0\0\0\0\0\0\0>

42.

<dev\0\0\0\0\0>

43.

<bin\0\0\0\0\0>

44.

<etc\0\0\0\0\0>

45.

<usr\0\0\0\0\0>

46.

<tmp\0\0\0\0\0>

9:

/ device directory

42.

140016

.byte 2,1

9f-.-2

41.

<..\0\0\0\0\0\0>

42.

<.\0\0\0\0\0\0\0>

01.

<tty\0\0\0\0\0>

02.

<ppt\0\0\0\0\0>

03.

<mem\0\0\0\0\0>

04.

<rf0\0\0\0\0\0>

05.

<rk0\0\0\0\0\0>

06.

<tap0\0\0\0\0>

07.

<tap1\0\0\0\0>

08.

<tap2\0\0\0\0>

09.

<tap3\0\0\0\0>

10.

<tap4\0\0\0\0>

11.

<tap5\0\0\0\0>

12.

<tap6\0\0\0\0>

13.

<tap7\0\0\0\0>

14.

<tty0\0\0\0\0>

15.

<tty1\0\0\0\0>

16.

<tty2\0\0\0\0>

17.

<tty3\0\0\0\0>

18.

<tty4\0\0\0\0>

19.

<tty5\0\0\0\0>

20.

<tty6\0\0\0\0>

21.

<tty7\0\0\0\0>

22.

<lpr\0\0\0\0\0>

01.

<tty8\0\0\0\0> / really tty

9:

/ binary directory

43.

140016

.byte 2,3

9f-.-2

41.

<..\0\0\0\0\0\0>

43.

<.\0\0\0\0\0\0\0>

9:

/ etcetra directory

44.

140016

.byte 2,3

9f-.-2

41.

<..\0\0\0\0\0\0>

44.

<.\0\0\0\0\0\0\0>

47.

<init\0\0\0\0>

9:

/ user directory

45.

140016

.byte 2,1

9f-.-2

41.

<..\0\0\0\0\0\0>

45.

<.\0\0\0\0\0\0\0>

9:

/ temporary directory

46.

140017

.byte 2,1

9f-.-2

41.

<..\0\0\0\0\0\0>

46.

<.\0\0\0\0\0\0\0>

9:

/ initialization program

47.

100036

.byte 1,3

9f-.-2

8:

sys    break; 0

sys    open; 6f-8b+core; 0

mov    r0,r1

sys    seek; 65.; 0

1:

mov    r1,r0

sys    read; 9f-8b+core; 512.

mov    9f,r5            / size

beq    1f

sys    creat; 9f-8b+core+4; 0

mov    r0,r2

movb   9f+2,0f

sys    chmod; 9f-8b+core+4; 0:..

movb   9f+3,0f

sys    chown; 9f-8b+core+4; 0:..

2:

tst    r5

beq    2f

mov    r1,r0

sys    read; 9f-8b+core; 512.

mov    $512.,0f

cmp    r5,$512.

bhi    3f

mov    r5,0f

3:

mov    r2,r0

sys    write; 9f-8b+core; 0:..

sub    r0,r5

br     2b

2:

mov    r2,r0

sys    close

br     1b

1:

mov    r1,r0

sys    close

sys    exec; 5f-8b+core; 4f-8b+core

sys    exit

4:

5f-8b+core; 0

5:

</etc/init\0>

6:

</dev/tap0\0>

.even

9:

/ end of initialization data

0

.endif