/ u3 -- unix

tswap:

movb    u.uno,r1 / move users process number to r1

mov     $runq+4,r2 / move lowest priority queue address to r2

jsr     r0,putlu / create link from last user on Q to u.uno's user

swap:

mov     $300,*$ps / processor priority = 6

mov     $runq,r2 / r2 points to runq table

1: / search runq table for highest priority process

tst     (r2)+ / are there any processes to run in this Q entry

bne     1f / yes, process 1f

cmp     r2,$runq+6 / if zero compare address to end of table

bne     1b / if not at end, go back

jsr     r0,idle; s.idlet+2 / wait for interrupt; all queues

/ are empty

br      swap

1:

tst     -(r2) / restore pointer to right Q entry

mov     r2,u.pri / set present user to this run queue

movb    (r2)+,r1 / move 1st process in queue to r1

cmpb    r1,(r2)+ / is there only 1 process in this Q to be run

beq     1f / yes

tst     -(r2) / no, pt r2 back to this Q entry

movb    p.link-1(r1),(r2) / move next process in line into

/ run queue

br      2f

1:

clr     -(r2) / zero the entry; no processes on the Q

2: / write out core to appropriate disk area and read in new process if

/ required

clr     *$ps / clear processor status

cmpb    r1,u.uno / is this process the same as the process in core?

beq     2f / yes, don't have to swap

mov     r0,-(sp) / no, write out core; save r0 (address in rout.

/ that called swap)

mov     sp,u.usp / save stack pointer

mov     $sstack,sp / move swap stack pointer to the stack pointer

mov     r1,-(sp) / put r1 (new process #) on the stack

tstb    u.uno / is the process # = 0

beq     1f / yes, kill process by overwriting

jsr     r0,wswap / write out core to disk

1:

mov     (sp)+,r1 / restore r1 to new process number

jsr     r0,rswap / read new process into core

jsr     r0,unpack / unpack the users stack from next to his program

/ to its normal

mov     u.usp,sp / location; restore stack pointer to new process

/ stack

mov     (sp)+,r0 / put address of where the process that just got

/ swapped in, left off., i.e., transfer control

/ to new process

2:

movb    $30.,uquant / initialize process time quantum

rts     r0 / return

wswap:

mov     *$30,u.emt / determines handling of emts

mov     *$10,u.ilgins / determines handling of illegal instructions

mov     u.break,r2 / put process program break address in r2

inc     r2 / add 1 to it

bic     $1,r2 / make it even

mov     r2,u.break / set break to an even location

mov     u.usp,r3 / put users stack pter at moment of swap in r3

cmp     r2,$core / is u.break less than $core

blos    2f / yes

cmp     r2,r3 / no, is (u.break) greater than stack pointer

bhis    2f / yes

1:

mov     (r3)+,(r2)+ / no, pack stack next to users program

cmp     r3,$ecore / has stack reached end of core

bne     1b / no, keep packing

br      1f / yes

2:

mov     $ecore,r2 / put end of core in r2

1:

sub     $user,r2 / get number of bytes to write out (user up

/ to end of stack gets written out)

neg     r2 / make it negative

asr     r2 / change bytes to words (divide by 2)

mov     r2,swp+4 / word count

movb    u.uno,r1 / move user process number to r1

asl     r1 / x2 for index

mov     r2,p.break-2(r1) / put negative of word count into the

/ p.break table

mov     p.dska-2(r1),r1 / move disk address of swap area for

/ process to r1

mov     r1,swp+2 / put processes dska address in swp +2 (block

/ number)

bis     $1000,swp / set it up to write (set bit 9)

jsr     r0,ppoke / write process out on swap area of disk

1:

tstb    swp+1 / is lt done writing?

bne     1b / no, wait

rts     r0 / yes, return to swap

rswap:

asl     r1 / process number x2 for index

mov     p.break-2(r1), swp+4 / word count

mov     p.dska-2(r1),swp+2 / disk address

bis     $2000,swp / read

jsr     r0,ppoke / read it in

1:

tstb    swp+1 / done

bne     1b / no, wait for bit 15 to clear (inhibit bit)

mov     u.emt,*$30 / yes move these

mov     u.ilgins,*$10 / back

rts     r0 / return

unpack: / move stack back to its normal place

mov     u.break,r2 / r2 points to end of user program

cmp     r2,$core / at beginning of user program yet?

blos    2f / yes, return

cmp     r2,u.usp / is break_above the "stack pointer before

/ swapping"

bhis    2f / yes, return

mov     $ecore,r3 / r3 points to end of core

add     r3,r2

sub     u.usp,r2 / end of users stack is in r2

1:

mov     -(r2),-(r3) / move stack back to its normal place

cmp     r2,u.break / in core

bne     1b

2:

rts     r0

putlu: / r1 = user process no.; r2 points to lowest priority queue

tstb    (r2)+ / is queue empty?

beq     1f / yes, branch

movb    (r2),r3 / no, save the "last user" process number in r3

movb    r1,p.link-1(r3) / put pointer to user on "last users" link

br      2f /

1:

movb    r1,-1(r2) / user is only user; put process no. at beginning

/ and at end

2:

movb    r1,(r2) / user process in r1 is now the last entry on

/ the queue

dec     r2 / restore r2

rts     r0

copyz:

mov     r1,-(sp) / put r1 on stack

mov     r2,-(sp) / put r2 on stack

mov     (r0)+,r1

mov     (r0)+,r2

1:

clr     (r1)+ / clear all locations between r1 and r2

cmp     r1,r2

blo     1b

mov     (sp)+,r2 / restore r2

mov     (sp)+,r1 / restore r1

rts     r0

idle:

mov	*$ps,-(sp) / save ps on stack

clr	*$ps / clear ps

mov	clockp,-(sp) / save clockp on stack

mov	(r0)+,clockp / arg to idle in clockp

1 / wait for interrupt

mov      (sp)+,clockp / restore clockp, ps

mov	(sp)+,*$ps

rts	r0

clear:

jsr       r0,wslot / get an I/O buffer set bits 9 and 15 in first

/ word of I/O queue r5 points to first data word

/ in buffer

mov     $256.,r3

1:

clr     (r5)+ / zero data word in buffer

dec     r3

bgt     1b / branch until all data words in buffer are zero

jsr	r0,dskwr / write zeroed buffer area out onto physical

/ block specified

rts     r0 / in r1