/ u4 -- unix

setisp:

mov     r1,-(sp)

mov     r2,-(sp)

mov     r3,-(sp)

mov     clockp,-(sp)

mov     $s.syst+2,clockp

jmp     (r0)

clock: / interrupt from 60 cycle clock

mov     r0,-(sp) / save r0

tst     *$lks / restart clock?

mov     $s.time+2,r0 / increment the time of day

inc     (r0)

bne     1f

inc     -(r0)

1:

mov     clockp,r0 / increment appropriate time category

inc     (r0)

bne     1f

inc     -(r0)

1:

mov     $uquant,r0 / decrement user time quantum

decb    (r0)

bge     1f / if less than 0

clrb    (r0) / make it 0

1: / decrement time out counts return now if priority was not 0

cmp     4(sp),$200 / ps greater than or equal to 200

bge     2f / yes, check time outs

tstb    (r0) / no, user timed out?

bne     1f / no

cmpb    sysflg,$-1 / yes, are we outside the system?

bne     1f / no, 1f

mov     (sp)+,r0 / yes, put users r0 in r0

sys     0 / sysrele

rti

2: / priority is high so just decrement time out counts

mov     $toutt,r0 / r0 points to beginning of time out table

2:

tstb    (r0) / is the time out?

beq     3f / yes, 3f (get next entry)

decb    (r0) / no, decrement the time

bne     3f / isit zero now?

incb    (r0) / yes, increment the time

3:

inc     r0 / next entry

cmp     r0,$touts / end of toutt table?

blo     2b / no, check this entry

mov     (sp)+,r0 / yes, restore r0

rti / return from interrupt

1: / decrement time out counts; if 0 call subroutine

mov     (sp)+,r0 / restore r0

mov     $240,*$ps / set processor priority to 5

jsr     r0,setisp / save registers

mov     $touts-toutt-1,r0 / set up r0 as index to decrement thru

/ the table

1:

tstb    toutt(r0) / is the time out for this entry

beq     2f / yes

decb    toutt(r0) / no, decrement the time

bne     2f / is the time 0, now

asl     r0 / yes, 2 x r0 to get word index for tout entry

jsr     r0,*touts(r0) / go to appropriate routine specified in this

asr     r0 / touts entry; set r0 back to toutt index

2:

dec     r0 / set up r0 for next entry

bge     1b / finished? , no, go back

br      retisp / yes, restore registers and do a rti

ttyi: / console tty input interrupt routine

jsr     r0,setisp / save reg r1, r2, r3

mov     *$tkb,r1 / r1 = char in tty reader buffer

inc     *$tks / set the reader enable bit

bic     $!177,r1 / clear upper 9 bits of the character (strip off

/ 8th bit of char)

cmp     r1,$'a-40 / is character upper case A,..., upper case Z.

/ note that

blt     1f / lower case a is represented by 141, upper case by

cmp     r1,$'z-40 / 101; and lower case z by 172, upper

/ case Z by 132.

bgt     1f / if not upper case, branch

add     $40,r1 / if upper case, calculate the representation of its

/ lower case counter part

1:

cmp     r1,$175 / char = "}"? Note: may be quit char (fs)

beq     2f / yes 2f

cmp     r1,$177 / char = "del" ?

beq     2f / yes, 2f

jsr     r0,putc; 0 / put char in r1 on clist entry

br 1f

movb    r1,ttyoch / put char in ttyoch

jsr     r0,startty / load char in tty output data buffer

cmp     r1,$4 / r1 = "eot"

beq     1f / yes, 1f

cmp     r1,$12 / r1 = "lf"

beq     1f / yes 1f

cmpb    cc+0,$15. / are there less than 15 chars on the input list

blo     retisp / yes, return

1:

jsr     r0,wakeup; runq; 0 / no, wakeup the input process

br      retisp / return

2: / r1 = "}" or "delete" to get here

mov     tty+[ntty*8]-8+6,r2 / move console tty buffer address to r2

beq     2f / if 0, wakeall

movb    r1,6(r2) / move "}" or del into "interrupt char"

/ byte of buffer

2:

jsr     r0,wakeall / wakeup all sleeping processes

br      retisp / return

wakeall:

mov     $39.,0f / flll arg2 of wakeup call wlth 39

1:

jsr     r0,wakeup; runq+4; 0:.. / wakeup the processes in the

dec     0b                      / wait list; decrement arg2

bge     1b / if not done, go back

rts     r0

ttyo: / console typewriter output interrupt routine

jsr     r0,setisp / save registers

jsr     r0,startty / put a char on the console tty output buffer

br      retisp / restore registers

retisp:

mov     (sp)+,clockp / pop values before interrupt off the stack

mov     (sp)+,r3

mov     (sp)+,r2

mov     (sp)+,r1

mov     (sp)+,r0

rti     / return from interrupt

ppti: / paper tape lnput interrupt routine

jsr     r0,setisp / save registers

movb    pptiflg,r1 / place "pptiflg" in r1

jmp     *1f(r1) / jump to location speclfled by value of "pptiflg"

1:

retisp / file not open

1f / file just opened

2f / file normal

retisp / file not closed

1: / file just opened

tstb    *$prs+1 / is error bit set in prs

bge     1f / no

jsr     r0,pptito / place 10 in toutt entry for ppt input

br      retisp

1:

movb    $4,pptiflg / change "pptiflg" to indicate file "normal"

2:

jsr     r0,wakeup; runq+2; 2 / wakeup process for ppt input entry

/ in wlist

tstb    *$prs+1 / is error bit set

blt     1f / yes

mov     *$prb,r1 / place contents ppt read buffer in r1

jsr     r0,putc; 2 / place character in clist area for ppt input

br .+2 / temp / if no space in clist character lost

cmpb    cc+2,$50. / character count in clist area for ppt lnput

/ greater than or equal to 50

bhis    retisp / yes

inc     *$prs / no, set reader enable bit in prs

br      retisp

1:

movb    $6,pptiflg / set pptiflg to 6 to indicate error bit set

br      retisp

/lpto:

/      jsr     r0,setisp

/      jsr     r0,starlpt

/      br      retisp

ppto: / paper tape output interrupt routine

jsr r0,setisp / save registers

jsr r0,starppt / get next character from clist, and output

/ if possible

br retisp / pop register values from stack

/ starlpt:

/       cmpb   cc+5.,$100.

/       bhi    1f

/       jsr    r0,wakeup; runq+2; 5

/1:

/       tstb   *$lps

/       bge    1f

/       jsr    r0,getc; 5

/              br 1f

/       mov    r1,*$lpb

/       br     starlpt

/1:

/       rts    r0

startty: / start or restart console tty output

cmpb    cc+1,$5.

bhi     1f / branch to 1f when character count on tty (? input,

/ output) list is greater than 5.

jsr     r0,wakeup; runq+2; 1

1:

tstb    *$tps / test console output ready bit

bge     2f / branch if ready bit is clear

tstb    toutt+0 / is toutt for console a zero

bne     2f / if not; branch to 2f

movb    ttyoch,r1 / put character to be output in r1

bne     1f

jsr     r0,getc; 1 / if char is nul, get a char from console

/ output list

br 2f / if console output list is empty, branch to 2f

1:

clrb    ttyoch

mov     r1,*$tpb / put character in console output register

cmp     r1,$12 / is char a line feed

bne     1f

movb    $15,ttyoch / put a cr in ttyoch

1:

cmp     r1,$11 / char = ht

bne     1f

movb    $15.,toutt+0 / set time out to 15 clock tics

1:

cmp     r1,$15 / char = cr

bne     2f

movb    $15.,toutt+0 / set time out to 15 clock ticks

2:

rts     r0

pptito: / paper tape input touts subrouting

cmpb    pptiflg,$2 / does "pptiflg" indicate file just opened

bne     1f / no, do nothing pyf

movb    $10.,toutt+1 / yes, place 10 in tout entry for tty input

tstb    *$prs+1 / is error bit set

blt     1f / yes, return

inc     *$prs / no, set read enable bit

1:

rts     r0

starppt: / start ppt output

cmpb    cc+3,$10. / is character count for ppt output greater

/ than 10.

bhi     1f / yes, branch

jsr     r0,wakeup; runq+2; 3 / no, wakeup process in wlist

/ entry for ppt input

1:

tstb    *$pps / is ready bit set in punch status word

bge     1f / no, branch

jsr     r0,getc; 3 / yes, get next char in clist for pptout and

/ place in r1

br      1f / if none, branch

mov     r1,*$ppb / place character in ppt buffer

1:

rts r0

wakeup: / wakeup processes waiting for an event by linking them to the

/ queue

mov     r1,-(sp) / put char on stack

mov     (r0)+,r2 / r2 points to a queue

mov     (r0)+,r3 / r3 = wait channel number

movb    wlist(r3),r1 / r1 contains process number in that wait

/ channel that was sleeping

beq     2f / if 0 return, nothing to wakeup

cmp     r2,u.pri / is runq greater than or equal to users process

/ priority

bhis    1f / yes, don't set time quantum to zero

clrb    uquant / time quantum = 0

1:

clrb    wlist(r3) / zero wait channel entry

jsr     r0,putlu / create a link from the last user on the Q

/ to this process number that got woken

2:

mov     (sp)+,r1 / restore r1

rts     r0

sleep: / wait for event

jsr     r0,isintr / check to see if interrupt or quit from user

br 2f / something happened / yes, his interrupt so return

/ to user

mov     (r0)+,r1 / put number of wait channel in r1

movb    wlist(r1),-(sp) / put old process number in there, on

/ the stack

movb    u.uno,wlist(r1) / put process number of process to put

/ to sleep in there

mov     cdev,-(sp) / nothing happened in isintr so

jsr     r0,swap / swap out process that needs to sleep

mov     (sp)+,cdev / restore device

jsr     r0,isintr / check for interrupt of new process

br      2f / yes, return to new user

movb    (sp)+,r1 / no, r1 = old process number that was originally

/ on the wait channel

beq     1f / if 0 branch

mov     $runq+4,r2 / r2 points to lowest priority queue

mov     $300,*$ps / processor priority = 6

jsr     r0,putlu / create link to old process number

clr     *$ps / clear the status; process priority = 0

1:

rts     r0 / return

2:

jmp     sysret / return to user

isintr:

mov     r1,-(sp) / put number of wait channel on the stack

mov     r2,-(sp) / save r2

mov     u.ttyp,r1 / r1 = pointer to buffer of process control

/ typewriter

beq     1f / if 0, do nothing except skip return

movb    6(r1),r1 / put interrupt char in the tty buffer in r1

beq     1f / if its 0 do nothing except skip return

cmp     r1,$177 / is interrupt char = delete?

bne     3f / no, so it must be a quit (fs)

tst     u.intr / yes, value of u.intr determines handling

/ of interrupts

bne     2f / if not 0, 2f. If zero do nothing.

1:

tst     (r0)+ / bump r0 past system return (skip)

4:

mov     (sp)+,r2 / restore r1 and r2

mov     (sp)+,r1

rts     r0

3: / interrupt char = quit (fs)

tst     u.quit / value of u.quit determines handling of quits

beq     1b / u.quit = 0 means do nothing

2: / get here because either u.intr <> 0 or u.qult <> O

mov     $tty+6,r1 / move pointer to tty block into r1

1: / find process control tty entry in tty block

cmp     (r1),u.ttyp / is this the process control tty buffer?

beq     1f / block found go to 1f

add     $8,r1 / look at next tty block

cmp     r1,$tty+[ntty*8]+6 / are we at end of tty blocks

blo     1b / no

br      4b / no process control tty found so go to 4b

1:

mov     $240,*$ps / set processor priority to 5

movb    -3(r1),0f / load getc call argument; character llst

/ identifier

inc     0f / increment

1:

jsr     r0,getc; 0:.. / erase output char list for control

br 4b / process tty. This prevents a line of stuff

/ being typed out after you hit the interrupt

/ key

br      1b